TYPE 3N

Model No: LE129-1(W/W),

LE129-2 (W/OR)

Power switch

Features

- 72pcs LED of LE129-1( Full brightness model / Energy saving model)

- 36pcs LED of white light and 36pcs LED of orange red light ( white/Orange Red version)

- LE129-2 ( white/ orange red) is for mosquito repellent

- Two sections operation

- Low power consumption and long life time

- 12V DC input voltage

- Design of power switch on the exterior

- Protection against lightning

- Water proof

- IP67

White/ Orange Red version                 <Up> energy saving model

<Down> Brightness model

Applications

- Ideal for interior lighting of the Truck bodies, campers and storage compartments.

- Equipped with mounting brackets for easy installation.

- Mosquito repellent of white/ Orange red version

- Water proof

Product Structure

| Main Components | Materials | Color |
| --- | --- | --- |
| Lens | PC | Mat White |
| Housing | Aluminum | Black |
| LED | 72 pcs LED | White (full brightness) |
|  | 36pcs/36pcs LED | White/orange red |

Drawing Dimension (Dimension: 355(L) x59(W) x67(H), Unit: mm)

Maximal Operation Values

| Description | Symbol | Value |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | LE129-1(W/W) | LE129-2(W/OR) |  |
| Voltage Input | VIN | 12 | 12 | V DC |
| Current Input | IIN | 350~640 mA | 305~295 mA |  |
| Power Dissipation | PD | 4/ 8 | 4 | W |
| Environmental Temperature | Tope | -30 ~ +40 |  | ℃ |
| Storage Temperature | Tsto | -40 ~ +80 |  | ℃ |

Illumination

| Description | Symbol | Typical | Unit |
| --- | --- | --- | --- |
| Luminous flux (White/white) <LE129-1> | F | 240(single) | lm |
|  |  | 450 (dual) |  |
| Luminous flux (White/orange) <LE129-2> | F | 210 (white) | lm |
|  |  | 15 (orange red) |  |

Installation

| Top View | Side View |
| --- | --- |

Mosquito Repellent Lighting Information:

(From light bulb technology blog)

“The human eye can only perceive a small band of wavelengths in the light spectrum, from about 390 to 750 nm. Insects perceive a similarly small band of the light spectrum, though their band of vision is shifted further to the “right” of the spectrum than ours. In fact, any wavelength higher than about 650 nm is virtually invisible to most flying insects.“